# MoboTerra ROS 2 user manual

Operator and vehicle interface edition   19 October 2026

Use operator/state and vehicle/mode for the implemented interface update. Selectable modes are MANUAL 0, AUTONOMOUS 1 and KINEMATICOFF 2. SAFETY 3 and ERROR 4 are automatic; UNKNOWN 255 indicates initial unknown readiness.

# Check the installed version before using updated commands

The newer local implementation is saved in the pre-pull update and is not currently integrated into the active checkout. A container rebuilt from that older checkout will not gain the new messages or modes. Verify OperatorState and VehicleModeState first. Source integration, a Jazzy build and secured hardware commissioning remain release requirements.

MoboTerra accepts either operator velocity, autonomous body velocity or autonomous wheel setpoints. The supervisor contains the mode state machine, checks readiness and faults, and publishes the only final wheel-motion command. The application reads vehicle/mode for operating state and operator/state for operator-device inputs. These messages have different responsibilities.

# Architecture and ownership

Names below are relative to /moboterra, the default robot namespace. Package and node names do not have to change when public topics are renamed. Raw joystick input remains joy; operator/state is semantic state, not a replacement name for a Joy message.

Operator velocity ─┐
Autonomy velocity ─┼─> Supervisor with vehicle mode state machine
Autonomy joints ───┘             │
                   velocity mode│           KINEMATICOFF
                                v                 │
                           Kinematics             │
                                │ candidate       │
                                └─> Supervisor <──┘
                                         │ final wheel array
                                         v
                                       Driver
                                         │ joint_states
                      Kinematics <───────┤
                           │             └─> robot_state_publisher
                           v
                vehicle/measured_velocity
                           │
                        Odometry ──> vehicle/odom

Safety, battery, wheel health and required component monitoring feed the supervisor. The supervisor publishes vehicle/mode to the application, operator provider and driver. Scanner field status, LaserScan, wheel feedback and odometry are also available to autonomy. The independent hardware safety chain remains responsible for physical safety; ROS monitoring is not safety-rated.

# Main topic reference

Types in the following table use ROS 2 package/msg/Type notation.

| Relative topic | Message type | Owner and purpose |
| --- | --- | --- |
| operator/cmd_vel | geometry_msgs/msg/TwistStamped | Operator provider; candidate body velocity |
| operator/state | mobotic_interfaces/msg/OperatorState | Operator provider; physical inputs and source validity |
| operator/mode_request | mobotic_interfaces/msg/VehicleModeRequest | Operator request for modes 0/1 only |
| autonomy/cmd_vel | geometry_msgs/msg/TwistStamped | External autonomy; candidate for AUTONOMOUS |
| autonomy/joint_setpoints | sensor_msgs/msg/JointState | External autonomy; candidate for KINEMATICOFF |
| vehicle/cmd_vel | geometry_msgs/msg/TwistStamped | Supervisor to kinematics; internal candidate |
| kinematics/joint_setpoints | sensor_msgs/msg/JointState | Kinematics back to supervisor; not directly to driver |
| wheel_modules/command | mobotic_interfaces/msg/WheelModuleCommandArray | Supervisor only; final motion for all four modules |
| wheel_modules/operation_command | mobotic_interfaces/msg/WheelModuleOperationCommand | Supervisor; internal enable/reset transaction |
| vehicle/mode | mobotic_interfaces/msg/VehicleModeState | Supervisor; effective mode, selected mode and readiness |
| joint_states | sensor_msgs/msg/JointState | Driver; authoritative measured joint state |
| wheel_modules/status | mobotic_interfaces/msg/WheelModuleStatusArray | Driver; steering/traction health and current |
| vehicle/measured_velocity | geometry_msgs/msg/TwistStamped | Kinematics; measured body velocity |
| vehicle/odom | nav_msgs/msg/Odometry | Odometry; wheel-derived pose and twist |
| battery/state | sensor_msgs/msg/BatteryState | Public battery telemetry, SOC as a fraction |
| battery/system_state | mobotic_interfaces/msg/BatterySystemState | Battery monitor; aggregate pack readiness/faults |
| safety/state | mobotic_interfaces/msg/SafetyState | Safety monitor; software motion permission and stop state |
| safety/io_state | mobotic_interfaces/msg/SafetyIOState | Safety monitor; mapped I/O and known flags |
| diagnostics | diagnostic_msgs/msg/DiagnosticArray | Component details for root-cause investigation |

The driver publishes no separate scalar wheel-motion topics. Consumers use the aggregated samples. Old manual/*, vehicle/mode_state, agv_vel and odometry names are not aliases in this updated implementation.

# Operating modes

| Value | Mode | Behavior |
| --- | --- | --- |
| 0 | MANUAL | Selects operator/cmd_vel; requires fresh connected and rearmed input |
| 1 | AUTONOMOUS | Selects autonomy/cmd_vel; ignores autonomy wheel candidates |
| 2 | KINEMATICOFF | Selects autonomy/joint_setpoints; velocity candidates have no motion effect |
| 3 | SAFETY | Automatic confirmed safety-chain stop, never selected by an application |
| 4 | ERROR | Automatic required-system fault; latched until verified explicit reset |
| 255 | UNKNOWN | Startup monitoring/readiness has not yet been established |

Only 0/1/2 are selectable. AUTO_VELOCITY and AUTO_DIRECT remain numeric aliases for 1 and 2. A mode change blocks candidates, stops through the outgoing path and checks fresh body and all-joint standstill. Default proof duration is 0.25 s; limits are 0.02 m/s body translation, 0.02 rad/s yaw and 0.1 rad/s per joint. A 10 s timeout latches ERROR. Fresh commands are required after a completed transition; a mode request never starts motion by itself.

current_mode is effective state. selected_mode retains the intended control mode during SAFETY/ERROR. ERROR takes precedence if both are present, while safety_stop_active preserves the overlapping safety-stop indication. Read error_active, reason_codes, motion_permitted, transition_in_progress, readiness_known, ready_to_drive and drives_enabled as well.

SAFETY includes confirmed protective stops, physical E-stop and known STO or safety-enable stops from mapped inputs. Missing monitoring is not proof of a confirmed safety stop: required monitoring loss becomes ERROR after the startup grace. Required drive faults, invalid position continuity, required component health failures and mode-transition timeout also produce ERROR. Not every diagnostic warning is a latched system error.

# Operator inputs and recovery

OperatorState contains source_id, connected, command_active, deadman_pressed, deadman_state_known, estop_pressed, estop_state_known, controls_neutral, turbo_active and status_message, with the acquisition stamp in header. Timer republication must not make an old input look new.

estop_pressed refers to the physical E-stop on the operator device. It is not the scanner stop, an aggregated vehicle stop or a mode selection. Unknown inputs must not be interpreted as confirmed released buttons. The default USB gamepad reports estop_state_known=false; REMdevice requires a verified PDO mapping before its physical controls can be relied on.

After a protective stop clears, a fresh command is required. Manual control also requires neutral controls; a configured deadman must be released and then pressed again while neutral. The default USB profile has no deadman button assigned. System errors require a supervisor reset transaction in addition to cleared causes. Old motion is not replayed after reset or reconnection.

# Joint commands and actual feedback

The four modules are front_left, front_right, rear_left and rear_right. Their eight names end in _steering and _traction, for example front_left_steering and front_left_traction. Use message name values to associate arrays; do not assume a positional ordering from an earlier worksheet.

For actual joint_states, position contains steering angles and wheel rotation angles [rad], and velocity contains steering and traction rotational velocities [rad/s]. The driver synchronizes all eight joints within the configured SYNC window and preserves acquisition time. effort is empty; electrical motor current is not torque and is not placed in JointState.effort.

In KINEMATICOFF, the external JointState candidate must contain every configured joint exactly once with finite position/velocity arrays and empty effort. Steering position is a target [rad]; steering velocity is a nonnegative profile speed [rad/s]; traction velocity is signed [rad/s]. Traction position entries are not traction-position targets. No public motor-current control is exposed.

All velocity candidates require a valid current source stamp and the configured body frame, normally base_link. Empty/map/odom frame IDs, future/expired commands, incomplete joints and non-finite values are rejected. A relabeled map-frame vector is not transformed automatically. The supervisor validates kinematics candidates and emits one authority-tagged final array; the driver independently validates it. Applications must not publish on internal command topics. Production DDS permissions are still needed to enforce publisher roles.

# Wheel health current and retained position

Each wheel_modules/status module has separate steering and traction DriveStatus records. Use each drive's motor_current [A] with current_known, native/status/control mode known flags, fault values and status_stamp, fault_stamp, current_stamp. Module-level fields include initialized, position_valid, position_reason, feedback_fresh, enabled, reset ID and acknowledgement, plus brake/STO values and known flags.

Elmo actual current 0x6078 is scaled using fresh rated current 0x6075. Missing or stale rating yields unknown current, not guessed amperes. Missing physical brake/STO mappings remain unknown. Known flags and freshness must be checked before a zero fault or a numerical reading is treated as valid.

Traction position is read through Elmo at startup and cyclically using PDOs. There is no direct encoder connection/configuration step. The current physical profile deliberately inhibits enable and authoritative joint publication until the retained Elmo reference, scale and identity are commissioned. The virtual launch alone enables the synthetic source. Never set virtual_position_source on physical CAN to bypass this gate. The persistent directory and durable helper do not yet establish recovery across power loss or unpowered movement.

# Scanner information for autonomy

For front_left and rear_right, public relative topics are:

scanner/<location>/scan for LaserScan, plus configured native scanner outputs.

scanner/<location>/safety_zone_status for ScannerSafetyState.

scanner/<location>/diagnostics for per-scanner DiagnosticArray.

ScannerSafetyState provides communication, protective/warning clear state, OSSD state and corresponding known flags. The generic safety adapter derives fields from the mapped safety-controller I/O; field-set number is currently unknown. Protective stops and unavailable monitoring are distinguished. LaserScan presence does not establish a clear protective field or safety permission.

# Service reference

| Relative service | Type | Use |
| --- | --- | --- |
| vehicle/set_mode | mobotic_interfaces/srv/SetVehicleMode | Request MANUAL=0, AUTONOMOUS=1 or KINEMATICOFF=2 |
| wheel_modules/clear_errors | std_srvs/srv/Trigger | Supervisor-owned asynchronous, standstill-guarded fault reset |
| vehicle/reset_odometry | std_srvs/srv/Trigger | Reset wheel odometry only with fresh measured standstill |
| internal/driver/initialize_all_drives | std_srvs/srv/Trigger | Commissioning only; rejected while drives are enabled |
| internal/driver/disable_all_drives | std_srvs/srv/Trigger | Internal disable; not a persistent safety override |

For mode selection, inspect accepted and transition_started, then monitor vehicle/mode for the effective state and completion. The service response's current_mode contains selected mode, not the effective SAFETY/ERROR override.

For fault reset, Trigger success means accepted, not completed. Monitor reset_in_progress, reset_id, reset_result, error_active and reason codes. The supervisor waits for matching fresh driver acknowledgements, standstill and cleared root causes. A 5 s timeout stays latched. Resetting odometry does not reset an encoder or clear a system fault. Reinitialization and automatic enable retries do not implicitly clear errors.

# Live inspection and release checks

Use the sourced Docker-shell commands and service examples in the command reference (commands.md). The updated integrated bringup enables description publication by default; do not start another robot_state_publisher. Capture the actual graph with moboterra-gui rqt_graph while the target is running. A source-defined architecture is not a captured live ROS graph.

Verify that only the supervisor publishes wheel_modules/command, the driver does not subscribe directly to kinematics joint candidates, all eight measured joints are coherent, current units/known flags are correct, and safety/fault recovery never replays stale motion. Hardware commissioning remains necessary after source integration and a ROS 2 Jazzy build/test run.

# Installation operation and command examples

MoboTerra exposes operator and vehicle interfaces for source inputs, mode selection, readiness and fault recovery. This guide covers their implemented commands, including AUTONOMOUS, KINEMATICOFF and the automatic SAFETY/ERROR states.

Version check before use: the 19 October pull left the older code in the working directory. The newer implementation is preserved in local stash a87fcd5, including its untracked message files. It must be integrated and built before the updated ROS commands will exist in a container. Rebuilding the older checkout alone will not add these interfaces. No automatic fallback aliases for the old topic names are provided. See the source audit (interface_source_audit.md).

The setup commands in sections 1–5 describe the installer currently checked out at 09d982b on Ubuntu 24.04 ARM64. The saved interface update also changes the installer/Dockerfile for AMD64 and mounted configuration, but those deployment changes have not been merged with the newer software-only setup. Do not assume that this combined deployment is already available on either architecture.

All host commands below run in Ubuntu Bash, not Windows PowerShell. Start in the MoboTerra repository root (the directory containing install.sh, deploy/ and src/). No host ROS installation is needed for the Docker workflow.

# Install software without robot hardware

bash ./install.sh

No arguments means software-only setup. It installs missing prerequisites, builds versioned runtime/GUI/CAD images, runs tests and isolated virtual checks, and installs moboterra-gui and moboterra-tools. Internet access and an Ubuntu administrator password are needed. It does not create a new hardware service or configure CAN. It also does not stop an existing hardware service: this is not a way to disable a running robot.

Equivalent explicit command, including a chosen ROS domain:

sudo bash ./deploy/install.sh --software-only --ros-domain-id 42

# Test the installed runtime without adapters

On the Ubuntu host, load the installed software image selection, then run the isolated smoke check. This publishes no motion commands, mounts no hardware and does not use host networking:

MOBOTERRA_IMAGE="$(python3 -c \
  'import json; print(json.load(open("/opt/moboterra/software/release.json"))["runtime_image"])')"
sudo docker run --rm --network none \
  --env ROS_LOCALHOST_ONLY=1 --env ROS_DOMAIN_ID=142 \
  "$MOBOTERRA_IMAGE" \
  python3 /opt/moboterra-checks/virtual_smoke.py

Expect a successful exit and a PASS report covering eight joints, wheel status, odometry and diagnostics. This is a software test, not hardware commissioning.

For an interactive virtual stack, use the same image variable in that terminal:

sudo docker run --rm -it --name moboterra-virtual \
  --network host \
  --env ROS_DOMAIN_ID=142 --env ROS_LOCALHOST_ONLY=1 \
  "$MOBOTERRA_IMAGE" \
  ros2 launch mobotic_bringup moboterra.launch.py \
  stack_type:=virtual robot_name:=moboterra_test \
  start_manual_interface:=false

Keep this terminal open. In a second Ubuntu terminal:

sudo docker exec -it moboterra-virtual \
  /usr/local/bin/moboterra-entrypoint bash
ros2 node list
ros2 topic echo --once /moboterra_test/wheel_modules/status

The second terminal is now a sourced container shell; exit leaves it. Domain 142 and namespace /moboterra_test separate simulated feedback from the default hardware stack (domain 42, namespace /moboterra). On an Ubuntu desktop:

moboterra-gui env ROS_DOMAIN_ID=142 ROS_LOCALHOST_ONLY=1 rqt_graph

Run that GUI command on the host, not inside the container shell. Stop the foreground virtual stack with Ctrl+C. Virtual tests do not verify actual CAN, scanner protective stops, brakes, encoder recovery or physical E-stop behavior.

# Configure the real adapters without starting motion

Secure the platform and make the independent hardware stop available first. The software is not hardware-commissioned or safety-rated. Do not start a second hardware launch while the supervised container is running.

sudo bash ./deploy/install.sh --no-start

Follow the prompts, connecting one adapter at a time to identify its unique USB serial. --no-start still requires real adapter serials; use --software-only when they are unavailable. It installs the hardware release and leaves the service stopped and disabled, including on a previously configured installation.

For unattended provisioning, replace both placeholders with actual unique serials:

sudo bash ./deploy/install.sh --no-start \
  --platform-serial PLATFORM_SERIAL \
  --battery-serial BATTERY_SERIAL \
  --ros-domain-id 42

The host supervisor assigns the platform adapter to can0 at 500000 bit/s and the battery adapter to can1 at 500000 bit/s, listen-only. It owns link naming, configuration and container lifecycle; do not bring CAN up in the container. There is no automatic USB-port fallback when serials are missing or duplicated.

Scanner/safety Ethernet addresses and physical safety wiring must be configured separately. The installer does not change them. Review commissioning checks in the deployment manual (../deploy/README.md#acceptance-checks) before enabling motion.

# Hardware start stop and boot behavior

These commands are on the Ubuntu host, after hardware configuration exists:

sudo moboterra-ctl status
sudo moboterra-ctl can
sudo moboterra-ctl start
sudo moboterra-ctl stop
sudo moboterra-ctl restart

Start/restart can enable drives when readiness gates allow it. A ROS/container stop is not a physical emergency stop. start starts this session only; it does not enable boot startup after --no-start. To opt into boot startup explicitly:

sudo systemctl enable --now moboterra-supervisor.service

To stop now and prevent automatic startup on later boots:

sudo systemctl disable --now moboterra-supervisor.service

The full guided command sudo bash ./deploy/install.sh both installs and enables/starts the hardware service. Use it only when the secured robot is ready. Missing configured adapters keep ROS stopped; process discovery and a Docker healthy state alone do not prove that the robot is ready or safe to move.

# Logs CAN and adapter replacement

sudo moboterra-ctl logs --lines 200
sudo moboterra-ctl logs --follow
sudo docker logs --follow --tail 200 moboterra-robot
sudo journalctl -u moboterra-supervisor.service -n 200 --no-pager
ip -details -statistics link show can0
ip -details -statistics link show can1

moboterra-ctl logs --follow follows the host supervisor journal, not live container output. Use docker logs --follow for ROS stdout/stderr. Ctrl+C exits a log follower without stopping the robot. ROS files are under /var/lib/moboterra/ros-logs; Docker and ROS log retention are bounded.

To inspect the serial for a connected CAN interface (substitute its actual current name if it is not can0):

udevadm info --query=property --path=/sys/class/net/can0 \
  | grep -E 'ID_SERIAL_SHORT|ID_PATH'

For replacement adapters, stop first, rerun guided identification, then review configuration before starting again:

sudo moboterra-ctl stop
sudo bash ./deploy/install.sh --reconfigure-can --no-start
sudo moboterra-ctl can
# Only after review and safe hardware commissioning:
sudo systemctl enable --now moboterra-supervisor.service

The current service is moboterra-supervisor.service, not robot-can.service. Configuration is /etc/moboterra/moboterra.env, not /etc/default/robot-can.

# Inspect the running ROS system

On the host, enter the supervised hardware container:

sudo docker exec -it moboterra-robot \
  /usr/local/bin/moboterra-entrypoint bash

The entrypoint sources ROS Jazzy and the installed workspace. Plain docker exec ... bash does not automatically source either environment. Inside this container shell, first verify that this image contains the interface update:

ros2 interface show mobotic_interfaces/msg/OperatorState
ros2 interface show mobotic_interfaces/msg/VehicleModeState
ros2 interface show mobotic_interfaces/msg/WheelModuleCommandArray

VehicleModeState must include AUTONOMOUS=1, KINEMATICOFF=2, SAFETY=3, ERROR=4 and selected_mode. If OperatorState is missing or the state contains only AUTO_VELOCITY/AUTO_DIRECT, stop here: this is the older built image. Topic discovery alone is insufficient when a provider has not been launched. For an updated image, use:

ros2 node list
ros2 topic list -t
ros2 service list -t
ros2 topic info --verbose /moboterra/joint_states
ros2 topic echo --once /moboterra/operator/state
ros2 topic echo --once /moboterra/vehicle/mode
ros2 topic echo --once /moboterra/wheel_modules/status
ros2 topic echo --once /moboterra/safety/state
ros2 topic echo --once /moboterra/safety/io_state
ros2 topic echo --once /moboterra/battery/system_state
ros2 topic hz /moboterra/joint_states
ros2 topic echo --once /moboterra/vehicle/measured_velocity
ros2 topic echo --once /moboterra/vehicle/odom

Some commands wait for a publisher; use Ctrl+C to leave them. Default robot namespace is /moboterra; adapt it when launching a differently named robot. Additional useful feedback: /moboterra/battery/state, /moboterra/scanner/front_left/scan and /moboterra/scanner/rear_right/scan. Inspect types with ros2 topic list -t rather than assuming all status messages are standard ROS messages.

To run one ROS command directly from the host:

sudo docker exec moboterra-robot \
  /usr/local/bin/moboterra-entrypoint \
  ros2 topic echo --once /moboterra/vehicle/mode

## Current modes and services

The mode state machine stays inside `mobotic_supervisor`; vehicle/mode is its published state, not a separate controller node. Only 0, 1 and 2 can be selected. A mode request does not bypass readiness gates or command motion.

| Value | Mode | Selection and command path |
| --- | --- | --- |
| 0 | MANUAL | Selectable; operator/cmd_vel through kinematics |
| 1 | AUTONOMOUS | Selectable; autonomy/cmd_vel through kinematics |
| 2 | KINEMATICOFF | Selectable; autonomy/joint_setpoints, bypassing kinematics |
| 3 | SAFETY | Automatic confirmed safety-chain stop; cannot be selected |
| 4 | ERROR | Automatic required-system fault, latched until verified reset |
| 255 | UNKNOWN | Initial readiness/monitoring not established |

AUTO_VELOCITY=1 and AUTO_DIRECT=2 remain numeric aliases, not additional modes. ERROR has display precedence over SAFETY; safety_stop_active still exposes a simultaneous stop. selected_mode retains the intended 0/1/2 control mode. current_mode is the effective state. Read motion_permitted, transition_in_progress, error_active, reason_codes, readiness_known, ready_to_drive and drives_enabled before interpreting an operating state.

Inside the sourced container shell, inspect the request fields first:

ros2 interface show mobotic_interfaces/srv/SetVehicleMode
ros2 service call /moboterra/vehicle/set_mode \
  mobotic_interfaces/srv/SetVehicleMode '{requested_mode: 0}'

For AUTONOMOUS or KINEMATICOFF, request the corresponding value on secured hardware or the isolated virtual stack. These are alternative requests, not a sequence to run automatically:

# AUTONOMOUS: external body velocity
ros2 service call /moboterra/vehicle/set_mode \
  mobotic_interfaces/srv/SetVehicleMode '{requested_mode: 1}'
# KINEMATICOFF: external steering position and traction velocity
ros2 service call /moboterra/vehicle/set_mode \
  mobotic_interfaces/srv/SetVehicleMode '{requested_mode: 2}'
ros2 topic echo /moboterra/vehicle/mode

The service returns accepted, transition_started, current_mode and message. Its current_mode is the selected control mode in the callback; use the topic for authoritative effective SAFETY/ERROR/UNKNOWN state and transition completion. /moboterra/operator/mode_request accepts timestamped requests for 0/1 only. Use the service for mode 2. Requests for 3/4/255 are rejected.

A transition stops the outgoing path and verifies fresh measured standstill for 0.25 s. A 10 s timeout latches ERROR. In KINEMATICOFF, operator/autonomy velocity candidates and kinematics output have no motion effect. Returning to a velocity mode requires the same verified transition and a new command.

## Fault recovery and odometry reset

Investigate reason_codes and diagnostics and remove the cause first. On a secured, stationary robot, the supervisor-owned reset request is:

ros2 service call /moboterra/wheel_modules/clear_errors \
  std_srvs/srv/Trigger '{}'
ros2 topic echo /moboterra/vehicle/mode

success: true means accepted, not completed. Follow reset_in_progress, reset_id, reset_result and error_active; completion requires fresh matching driver acknowledgements, standstill and cleared root faults. A 5 s timeout leaves the fault latched. Reinitializing/enabling does not implicitly clear it. Successful recovery clears cached motion and still requires a fresh command.

A confirmed safety stop clears without a fault reset only if no separate fault is latched. Motion then requires a new command. Manual operation also requires neutral controls; if configured, release and press the deadman again. The USB profile has no deadman assigned by default.

Odometry has a separate standstill-guarded reset; this is not an encoder reset:

ros2 service call /moboterra/vehicle/reset_odometry \
  std_srvs/srv/Trigger '{}'

The remaining driver commissioning services are internal, not normal operator controls. Both use std_srvs/srv/Trigger: internal/driver/initialize_all_drives and internal/driver/disable_all_drives. The old public enable/clear service commands are not part of the updated contract. Do not use initialization or internal disable as fault-reset or persistent stop workarounds.

The external motion inputs are operator/cmd_vel, autonomy/cmd_vel and autonomy/joint_setpoints under the robot namespace. Do not publish directly to internal driver/kinematics paths to bypass arbitration. wheel_modules/command now carries the single final motion array, published only by the supervisor. Management/enable/reset transactions moved to internal wheel_modules/operation_command. Kinematics publishes a candidate back to the supervisor, never directly to the driver. These changes are not simple renames.

## Operator state and hardware feedback

operator/state reports physical/source inputs, not the vehicle operating mode. estop_pressed means the physical operator-device button; read it only together with estop_state_known. A USB gamepad reports the E-stop as unknown, not a verified released safety button. Raw joy remains joy. The vehicle's mode aggregates operator, safety-chain and system-fault conditions separately.

joint_states is the authoritative eight-joint measurement: steering and wheel position [rad], steering and wheel velocity [rad/s]; effort is empty. wheel_modules/status contains per-module steering and traction drive status, with separate motor_current [A] and current_known, status/fault/mode fields and acquisition stamps. It also exposes position_valid, position_reason, freshness, enable, reset, brake and STO known flags. Unknown values are not zero-valued proof of health.

The updated driver's physical profile presently inhibits enable and authoritative joint publication until the retained Elmo position reference and scale are commissioned. It reads traction position through Elmo (startup SDO and cyclic position PDO), not a direct encoder connection. Only virtual launch sets virtual_position_source=true; never use it on physical CAN. A persistent directory/helper alone does not recover unpowered wheel movement.

Scanner field monitoring is available to autonomy as well as the supervisor:

ros2 topic echo --once /moboterra/scanner/front_left/safety_zone_status
ros2 topic echo --once /moboterra/scanner/rear_right/safety_zone_status
ros2 topic echo --once /moboterra/scanner/front_left/diagnostics
ros2 topic echo --once /moboterra/scanner/rear_right/diagnostics

Check communication and known flags. Field/OSSD state comes from the mapped safety-controller I/O; field-set number is currently unknown. Protective stop and monitoring failure are different conditions. LaserScan does not establish protective-stop permission. See the ROS interface manual (ros_interface_manual.md).

# GUI tools and live graph

On the host desktop, as the logged-in user:

moboterra-gui rviz2
moboterra-gui rqt
moboterra-gui rqt_graph

X11/XWayland, DISPLAY and a readable Xauthority file are required. Do not prefix the launcher with sudo; it handles Docker authentication. In rqt_graph, refresh and adjust hidden-topic filters to capture the actual running graph. A source-derived architecture figure is not a live graph capture.

In the updated bringup, start_description defaults to true, so robot_state_publisher is integrated. Do not start a duplicate. RViz needs the namespaced TF topics:

moboterra-gui rviz2 --ros-args \
  -r /tf:=/moboterra/tf -r /tf_static:=/moboterra/tf_static

For the older checkout only, description was a separate optional launch. Verify the running nodes before applying instructions from that version. Software GL is the default; optional DRI access is enabled with MOBOTERRA_GUI_DRI=1 moboterra-gui rviz2 on compatible hosts.

# Edit code pull updates and deploy a tested release

Yes, code is editable. Edit the repository on the host, not files inside the running container: the deployed workspace is a built image, not a live source mount. Keep local changes committed or backed up. Before pulling:

git status --short
# Continue only when your changes are safely handled and the checkout is clean:
git pull --ff-only

Do not blindly apply/drop an existing stash. On this development Windows checkout the newer interface implementation remains in stash a87fcd5; its untracked parent contains essential messages and helpers. Integration must include those files and resolve deployment conflicts, not just rename topics. Verify section 6's message definitions in the rebuilt container before testing.

Pulling or editing source does not update an existing Docker image. After reviewing changes, rebuild and run the software tests from the repository root:

bash ./install.sh

Then run the isolated virtual check in section 2. On an existing hardware installation this software-only build does not select the new image for the hardware service. During a secured maintenance window, deploy it separately:

sudo moboterra-ctl stop
sudo bash ./deploy/install.sh --no-start
# Review configuration and commissioning results before startup:
sudo systemctl enable --now moboterra-supervisor.service
sudo moboterra-ctl status
sudo moboterra-ctl logs --lines 200

The saved updated Compose file mounts /etc/moboterra read-only in the container and passes /etc/moboterra/platform.yaml as platform_config. It also mounts /var/lib/moboterra/position-state as writable state storage. Policy overrides must name existing files under /etc/moboterra; their env keys include DRIVER_PARAMETERS_FILE, SUPERVISOR_PARAMETERS_FILE and MANUAL_PARAMETERS_FILE. Code changes still need an image rebuild. Platform or policy edits need validation and a supervised restart.

Those mounts are not present in the pulled checkout's Compose file. Until deployment changes are integrated, edit the checkout YAML and rebuild/redeploy; editing a host file alone has no effect. Verify the installed container mounts and image rather than inferring them from a directory's existence:

sudo docker inspect moboterra-robot \
  --format 'image={{.Config.Image}} mounts={{json .Mounts}}'

The installed software release's release.json records its runtime image; software.env contains GUI/tool selections and the ROS domain, not a MOBOTERRA_IMAGE entry. Installed hardware settings/image selections are in /etc/moboterra/moboterra.env; software-only selections are in /etc/moboterra/software.env. GUI prefers hardware settings when present; offline tools use software settings. Do not store secrets in these files.

If a previous installed hardware release exists, and rollback is safe:

sudo moboterra-ctl rollback

Rollback swaps the installed current/previous image selections and restarts hardware. It does not revert Git source, firmware, adapter identity or calibration. Inspect logs and recheck hardware after either upgrade or rollback.

# Offline CAD preview

On the host, in a writable output directory:

moboterra-tools

This writes moboterra-preview.png in the current directory. To inspect a provided STEP assembly, place it in that same directory first:

moboterra-tools python /opt/moboterra-tools/inspect_scanner_datums.py \
  MoboTerra_assembly.step

The tools container uses the caller's user ID, mounts the current directory and has no network. The installer does not supply the STEP source file.